# MATLAB Quiz

| Student Name |  |  | ID |  |
| --- | --- | --- | --- | --- |

## This assignment covers ABET outcomes 1,2, and 5.

## The script contains 10 intentional mistakes. Identify and correct all mistakes, highlight your corrections in the code (Section 2), then upload your corrected version using the provided upload link.

## Section 1: Original Wrong Code

%=========================================================================% % QUIZ: Control Systems % % There are 10 mistakes in this code. Find and fix them! % %=========================================================================% clc; clear; %% Define System 1 A1 = [0 1 0; 0 0 1; -6 -11 -6]; B1 = [0 0 1]; C1 = [1; 0; 0]; D1 = 0; sys1 = ss(A1, B1, C1, D1); %% Define System 2 A2 = [0 1 0; -2 -3 0; 0 1 -4]; B2 = [1 0 0]; C2 = [0; 1; 1]; D2 = [0]; sys2 = ss(A2, B2, C2, D2); %% 1. Check controllability Co1 = ctrb(sys1); Co2 = ctrb(A2, B2); rank_C1 = rank(Co1); rank_C2 = rank(Co2); disp('Rank of controllability matrix System 1:'), disp(rank_C1) disp('Rank of controllability matrix System 2:'), disp(rank_C2) %% 2. Check observability Ob1 = obsv(sys1); Ob2 = obsv(A2, C2); rank_O1 = rank(Ob1); rank_O2 = rank(Ob2); disp('Rank of observability matrix System 1:'), disp(rank_O1) disp('Rank of observability matrix System 2:'), disp(rank_O2) %% 3. Check minimal realization sys1_min = minreal(sys1); sys2_min = minreal(A2, B2, C2, D2); %% 4. Step responses figure; step(sys1, sys2) legend('System 1', 'System 2') title('Step Response Comparison') % 6. Observability comparison comment (students analyze) if rank_C1 = rank(A1) disp('System 1 is fully controllable'); else disp('System 1 is NOT controllable'); end

## Section 2: Corrected Code with highlights